# Biondi-Style Broad-Overlap
Sub-Aperture Pixel-Tracking Tomography

Full Mathematical Development of the Working Protocol

Dual modal branches: full-aperture m = 1–24 and selective W = 25 / m = 1–6

Status. This document is a mathematical expansion of Protocol v1.4 plus the later low-order ellipse-gated branch developed in the Great Pyramid experiments. The preprocessing, two-arc nuisance correction, Kz geometry, and W=25 real two-component depth inversion are the canonical core. The full m=1–24 modal characterization is the canonical modal-analysis branch from v1.4. The m=1–6 W25 ellipse gate is a newer selective experimental branch and should remain explicitly labeled exploratory until independently validated.

Epistemic constraint. The measured vectors are image-registration observables. They are not automatically literal wall displacement or physical particle motion. Candidate depth is conditional on the adopted effective wavelength/velocity model and on the physical adequacy of the vector forward model.

Source basis: Generic Biondi-Style Sub-Aperture Tomography Protocol v1.4, its companion JSON configuration, and the current W25 ellipse-gating implementation used in the Great Pyramid branch.

# Contents

1. Protocol status and branch architecture

2. Symbols, indices, and coordinate conventions

3. Complex SLC and broad-overlap Master/Offset construction

4. Complex subpixel co-registration and target vector Y

5. Canonical two-arc robust nuisance correction

6. Scalar coherence quality-control branch

7. Branch A — full-aperture modal analysis, m = 1–24

8. Virtual-baseline geometry and Kz

9. Canonical W = 25 real two-component depth inversion

10. Branch B — selective W25 ellipse gate, m = 1–6

11. Controls, recurrence, signed-depth equivalence, and ghosts

12. Current frozen implementation values

13. End-to-end algorithms for the two branches

14. Interpretation and validation gates

Appendix A. Linear-algebra derivations

Appendix B. Relationship between modal ellipses and depth ellipses

Appendix C. Required output package and reproducibility manifest

# 1. Protocol status and branch architecture

The protocol now has one common measurement and inversion core with two modal branches. The branches diverge only after the nuisance-corrected two-component vector sequence q has been formed. They answer different questions and should not be conflated.

| Element | Branch A: full modal 1–24 | Branch B: selective modal 1–6 |
| --- | --- | --- |
| Aperture support for modal fit | Full ordered K=50 sequence | Each contiguous W=25 window |
| Allowed modal orders | m=1,…,24 | m=1,…,6 |
| Purpose | Describe the complete positive-harmonic modal fingerprint of q | Reject W25 vector families that are not sufficiently elliptical, low-order, and non-degenerate |
| Modal intercept | No intercept in the frozen K50 modal metrics | 2D intercept c is fitted in each W25 family |
| Primary modal score | R² and exploratory ellipse score R²(b/a) | Adjusted R² of the best m in 1–6, then axis-ratio and minor-axis gates |
| Depth inversion | Canonical ungated W25 real vector LS; modal metrics accompany interpretation | Only accepted W25 families are passed to the same real vector LS depth inversion |
| Current status | Canonical modal characterization from v1.4 | Exploratory selective branch; not yet canonical replacement |

The common core is

SLC → DFT₂ → broad-overlap Master/Offset pairs → 32×32 complex u-PCC → Yₖ → two-arc G_arc,ₖ → qₖ = Yₖ − G_arc,ₖ    (1)

After Eq. (1), Branch A computes the full K50 modal spectrum while the canonical depth inversion uses every contiguous W25 support. Branch B instead fits only low-order ellipses inside each W25 support and allows a support into the depth inversion only when it satisfies the current ellipse gate.

# 2. Symbols, indices, and coordinate conventions

| Symbol | Meaning |
| --- | --- |
| x | Ordered spatial sample on the frozen target transect; often represented by line position P. |
| k | Matched broad-overlap sub-aperture-pair index; k=0,…,K−1, with K=50 in the current implementation. |
| w | Start index of a contiguous depth-support window; for K=50 and W=25, w=0,…,25. |
| n | Local index inside a W25 window; n=0,…,24. |
| m | Modal order in the ordered pair-index coordinate. It is a harmonic index, not automatically a physical cavity eigenmode. |
| Yₖ(x) | Raw 2D target registration vector [dx,dy]. |
| G_arc,ₖ | Two-arc robust common-mode/nuisance vector. |
| qₖ(x) | Canonical corrected vector observable Yₖ−G_arc,ₖ. |
| Kz,ₖ | Depth steering wavenumber assigned to pair k. |
| z | Candidate unsigned depth coordinate of the inversion. |
| Q | Matrix whose rows are 2D q vectors. |
| ¦¦·¦¦F | Frobenius norm. |

Canonical array convention is A[azimuth,row ; range,column]. Display coordinates may be written as (column,row), but saved processing tables must name row and column explicitly. dx is a column shift and dy is a row shift. The current Great Pyramid line coordinate P increases upward from the ground end of the rasterized line.

# 3. Complex SLC and broad-overlap Master/Offset construction

Let s[a,r] be the complex SLC with azimuth-like index a and slant-range-like index r. The canonical spectral construction is the two-dimensional discrete Fourier transform

S̃[p,q] = Σₐ Σᵣ s[a,r] exp{−i2π(pa/Nₐ + qr/Nᵣ)}.    (2)

Only the azimuth/Doppler-conjugate coordinate is partitioned into the broad overlapping pair masks, but the canonical implementation is expressed as DFT2 followed by two-dimensional masking and IDFT2. A reduced-dimensional implementation is permitted only after numerical equivalence to DFT2 has been demonstrated on representative data.

For pair k define two broad masks M_M,k and M_O,k (“Master” and “Offset”; the v1.4 document used “Reference” and “Offset”). Let the mask centers be c_M,k and c_O,k with fixed separation

|c_O,k − c_M,k| = B_shift.    (3)

cₖ = ½(c_M,k + c_O,k),    (4)

where cₖ is the ordered pair center swept across the available processed Doppler support. For a rectangular mask of bandwidth B_RO,

M_M,k(f) = 1{|f−c_M,k| ≤ B_RO/2},    M_O,k(f) = 1{|f−c_O,k| ≤ B_RO/2}.    (5)

The reconstructed complex sub-apertures are

Mₖ = DFT₂⁻¹{S̃ · M_M,k},    Oₖ = DFT₂⁻¹{S̃ · M_O,k}.    (6)

Important. These are broad, heavily overlapping sub-apertures, not narrow independent Doppler bins. Pair order k is preserved through registration, nuisance correction, modal analysis, and depth inversion.

# 4. Complex subpixel co-registration and target vector Y

For each target position x and pair k, extract matched 32×32 complex patches P_M,k(x) and P_O,k(x). The canonical estimator is the complex, unnormalized phase-cross-correlation implementation with normalization=None and subpixel upsampling factor 100.

Cₖ(δ) = F⁻¹{ F(P_M,k) · F(P_O,k)* }(δ),    (7)

δ̂ₖ = arg max_δ |Cₖ(δ)|,    (8)

with the peak locally upsampled to estimate subpixel translation. The observed target vector is

Yₖ(x) = [ dxₖ(x), dyₖ(x) ] ∈ ℝ².    (9)

Only Master_k is registered against Offset_k at the same k. Global registration of the entire look stack and registration of k against k+1 are not part of the protocol. Injected rigid-shift regression is required to verify sign, units, and subpixel scale.

# 5. Canonical two-arc robust nuisance correction

The former two-focal-flank correction is retired from the canonical path. The current nuisance estimator uses two spatial arcs lying on opposite sides of the target strip. Each arc contains J independently registered 32×32 supports, J=10 per side by default, with centers at least 40 pixels from the entire target strip.

For side s∈{+,-}, pair k, and arc sample j, let F_s,k^(j)∈ℝ² be the measured registration vector. The two-dimensional geometric median is

G_s,k = arg min_g Σⱼ ||F_s,k^(j) − g||₂.    (10)

The canonical regional nuisance vector and corrected target observable are

G_arc,k = ½(G_+,k + G_−,k),    (11)

qₖ(x) = Yₖ(x) − G_arc,k.    (12)

The side disagreement and within-side scatter are retained as quality-control variables, not discarded diagnostics:

D_arc,k = ||G_+,k − G_−,k||₂.    (13)

S_s,k = medianⱼ ||F_s,k^(j) − G_s,k||₂    (or the explicitly declared robust scatter statistic).    (14)

The identity Yₖ=G_arc,k+qₖ is verified component-by-component after serialization. Raw Y remains an unsubtracted control observable. The historical two-flank residual q_old is retained only for regression/audit comparisons.

# 6. Scalar coherence quality-control branch

Scalar complex coherence is maintained as an independent screening and data-quality observable, not as the vector depth-inversion input. For a spatial estimator Ω around x, a standard normalized complex coherence is

γₖ(x) = |Σ_{u∈Ω} Mₖ(u) Oₖ(u)*| / √[(Σ|Mₖ(u)|²)(Σ|Oₖ(u)|²)].    (15)

dₖ(x) = 1 − γₖ(x).    (16)

A seven-pixel spatial estimator/aggregation may be used in the current workflow. The pairwise values are saved before spatial aggregation. Scalar coherence and vector registration can respond differently to intensity morphing, layover/shadow, multipath, or nonrigid focus changes, so neither is substituted for the other.

# 7. Branch A — full-aperture modal analysis, m = 1–24

This branch characterizes the entire ordered K=50 corrected vector sequence at each target position. It does not use the W25 low-order gate. The frozen K50 modal metrics use a zero-intercept harmonic fit for each m=1,…,24.

For K=50 define

θₖ,m = 2πmk/K,    k=0,…,K−1.    (17)

D_m = [ cos θₖ,m   sin θₖ,m ]_{k=0}^{K−1}  ∈ ℝ^{K×2}.    (18)

Q(x) = [q₀(x)^T; … ; q_{K−1}(x)^T] ∈ ℝ^{K×2}.    (19)

The least-squares harmonic coefficient matrix is

B_m(x) = arg min_B ||Q(x) − D_m B||F² = D_m⁺ Q(x),    (20)

Q̂_m(x) = D_m B_m(x).    (21)

For the existing K50 branch the modal coefficient matrix is equivalently written

E_m = B_m^T = [[A_m, B_m],[C_m, D_m]],    (22)

so that

q_x(k) = A_m cos θₖ,m + B_m sin θₖ,m,    (23)

q_y(k) = C_m cos θₖ,m + D_m sin θₖ,m.    (24)

The stored modal R² is computed against the centered total vector energy even though the harmonic fit itself has no intercept:

R²_m = 1 − ||Q−Q̂_m||F² / ||Q−1 μ^T||F²,    μ = K⁻¹Σₖ qₖ.    (25)

The parametric locus E_m[cos θ, sin θ]^T is an ellipse (possibly degenerate). Let

E_m = U Σ V^T,    Σ = diag(a_m,b_m),    a_m ≥ b_m ≥ 0.    (26)

Then a_m and b_m are the major and minor semiaxes in registration-pixel units. The branch stores

ρ_m = b_m/a_m,    (27)

A_rms,m = ||E_m||F/√2 = √[(a_m²+b_m²)/2],    (28)

Score_ellipse,m = R²_m · ρ_m.    (29)

The major-axis orientation is the angle of the first column of U in the (dx,dy) plane. A convenient handedness sign is sign(det E_m); changing sign corresponds to reversal of traversal around the fitted ellipse.

For K=50, the real discrete spectrum consists of DC, positive harmonics m=1,…,24, the Nyquist mode m=25, and conjugate negative harmonics m=26,…,49. Restricting the full modal branch to m=1,…,24 therefore gives the non-DC, non-Nyquist positive-harmonic set without double counting conjugate partners. The dominant descriptive mode may be defined by

m* = arg max_{1≤m≤24} [R²_m (b_m/a_m)].    (30)

Interpretive caution. The modal order m is an index in ordered sub-aperture pair space. Calling m a physical mechanical mode requires an additional mapping from pair index to time/frequency and a validated physical forward model. The protocol stores m as a signal fingerprint, not as an automatic eigenfrequency label.

Branch A then proceeds to the canonical W25 depth inversion described in Section 9 without rejecting windows merely because their full-aperture dominant m is high or low. The m=1–24 metrics accompany the depth product as a full-aperture fingerprint.

# 8. Virtual-baseline geometry and Kz

Each ordered pair center cₖ is mapped, using acquisition metadata, to a virtual aperture state at time tₖ. Let r_s(t) be the sensor position and v_ref the velocity at the reference/aperture-center state. Let r_t be the target position used for the local geometry calculation.

e_LOS = (r_t − r_s(t_ref)) / ||r_t − r_s(t_ref)||₂,    (31)

e_A = [v_ref − (v_ref·e_LOS)e_LOS] / ||v_ref − (v_ref·e_LOS)e_LOS||₂,    (32)

Bₖ = r_s(tₖ) − r_s(t_ref),    (33)

B⊥,ₖ = Bₖ · e_A.    (34)

The operative baseline is therefore the same-orbit virtual inter-look baseline projected perpendicular to the line of sight in the aperture plane. It is not the tiny cross-orbit component e_LOS×e_A.

Kz,ₖ = 4π B⊥,ₖ / [ λ_eff rₖ sin θₖ ].    (35)

Here rₖ is the slant range, θₖ the declared incidence-angle convention, and λ_eff the wavelength used by the current depth model. In the present Great Pyramid working implementation λ_eff is an explicitly declared operational wavelength calibrated from the Campbell control; it is not the X-band radar carrier wavelength. This distinction must remain visible in every manifest.

Calibration rule. A site used to choose λ_eff is a calibration anchor. It cannot simultaneously be counted as an independent validation of absolute depth.

The numerical Kz implementation has passed an independent state-vector reconstruction audit at approximately 10⁻⁶ relative scale. This validates the implemented geometry, not the physical truth of λ_eff or of the image-registration-to-depth forward model.

# 9. Canonical W = 25 real two-component depth inversion

The depth inversion asks whether a contiguous portion of the corrected vector history lies in the two-dimensional cos/sin subspace generated by a candidate depth z. With K=50 and W=25 there are 26 contiguous windows.

w = 0,…,K−W,    I_w = {w,w+1,…,w+W−1}.    (36)

X_z,w(k,:) = [ cos(Kz,ₖ z),  sin(Kz,ₖ z) ],    k∈I_w.    (37)

Q_w(x) = [qₖ(x)^T]_{k∈I_w} ∈ ℝ^{W×2}.    (38)

B_z,w(x) = arg min_B ||Q_w(x) − X_z,w B||F² = X_z,w⁺ Q_w(x).    (39)

Q̂_z,w = X_z,w B_z,w.    (40)

The depth-fit coefficient matrix B_z,w has four scalar coefficients. The two columns are the fitted dx and dy responses to the cosine and sine steering columns. The unadjusted score is

R²_z,w = 1 − SSE_z,w / SST_w,    (41)

SSE_z,w = ||Q_w−Q̂_z,w||F²,    SST_w = ||Q_w−1 μ_w^T||F².    (42)

Because each W25 fit contains n=2W=50 scalar observations and p=4 scalar fitted coefficients, the canonical adjusted score is

adjR²_z,w = 1 − (1−R²_z,w)(n−1)/(n−p−1),    n=2W, p=4.    (43)

S_W25(x,z) = max_w adjR²_z,w(x).    (44)

The winning window identity w*(x,z) is saved. A candidate is stronger when nearby spatial samples show compatible depth support and when multiple overlapping W25 windows support the same family rather than one isolated support.

## 9.1 Why the fit is two-component and real

The physical candidate-depth fit preserves q as a genuine two-component vector. Collapsing q_x+i q_y into one complex scalar changes the model and is retained only as a signed-branch/handedness diagnostic. The real model allows the vector ellipse at candidate depth z to have arbitrary orientation and ellipticity through the fitted 2×2 coefficient matrix B_z,w.

## 9.2 z = 0 rank deficiency

At z=0, the steering columns become [1,0], so Eq. (37) has rank one. In the current ellipse-gated implementation, z=0 is therefore scored separately as a constant-vector diagnostic rather than being treated as an ordinary positive-depth fit:

Q_w ≈ 1 c^T,    c = W⁻¹Σ_{k∈I_w} qₖ,    (45)

with two fitted scalar constants. A strong z=0 score means that the W25 vector family is close to constant in pair index; it is not by itself proof of a literal surface reflector.

# 10. Branch B — selective W25 ellipse gate, m = 1–6

Branch B acts upstream of the depth scan. It asks whether each W25 vector family itself exhibits a sufficiently coherent, non-degenerate low-order ellipse in ordered pair-index space. Only accepted W25 families are permitted to vote in the depth tomogram.

For a given spatial position x and window start w, relabel the 25 vectors locally by n=0,…,W−1. For each m=1,…,6 define

ϑ_n,m = 2πmn/W.    (46)

q_n = c + u cos ϑ_n,m + v sin ϑ_n,m + ε_n,    (47)

where c,u,v∈ℝ². In matrix form,

H_m = [1, cos ϑ_n,m, sin ϑ_n,m]_{n=0}^{W−1} ∈ ℝ^{W×3},    (48)

C_m = arg min_C ||Q_w − H_m C||F².    (49)

C_m contains six scalar parameters: the 2D center c and four harmonic coefficients. The ellipse matrix is formed from u and v,

E_m = [u  v] ∈ ℝ^{2×2}.    (50)

E_m = U_m diag(a_m,b_m) V_m^T,    a_m≥b_m≥0.    (51)

ρ_m = b_m/a_m.    (52)

The fit score is computed against the centered vector energy. With n_s=2W=50 scalar observations and p=6 fitted scalar coefficients,

adjR²_ellipse,m = 1 − (1−R²_ellipse,m)(n_s−1)/(n_s−p−1),    p=6.    (53)

The winning low-order ellipse for that W25 family is selected by maximum adjusted ellipse R²:

m*_w = arg max_{m∈{1,…,6}} adjR²_ellipse,m.    (54)

The current exploratory acceptance gate is

adjR²_ellipse,m* ≥ 0.25,    b/a ≥ 0.10,    b ≥ 0.005 pixel.    (55)

Current gate definition. There is no phase-lock criterion in the present branch. Mode selection is by maximum adjusted ellipse R², not by the full-aperture ellipse score R²(b/a). The axis-ratio and minor-axis requirements are applied after choosing the best m in 1–6.

## 10.1 Why m ≤ 6

The W25 branch deliberately imposes a stricter sampling condition than discrete-time Nyquist. The number of vector samples per fitted circuit is W/m. Requiring at least approximately four samples per circuit gives

W/m ≥ 4  ⇒  m ≤ W/4 = 6.25  ⇒  m_max = 6.    (56)

Thus m=6 has 25/6≈4.17 vectors per circuit, whereas m=7 has only 3.57. Modes 7–11 are mathematically representable in W25 but fall below this conservative ellipse-geometry sampling criterion. The recent 7–11 stress test was therefore intentionally outside the 1–6 branch and is not one of the two principal branches specified here.

## 10.2 Gated depth score

Let A(x,w) be 1 when Eq. (55) passes and 0 otherwise. Branch B uses exactly the same depth fit in Eqs. (37)–(43), but only over accepted windows:

S_gate(x,z) = max_{w : A(x,w)=1} adjR²_z,w(x).    (57)

If no W25 family is accepted at x, the tomogram entry is missing/blank, not a numerical zero. At z=0, the separate constant-vector diagnostic in Eq. (45) is evaluated only over accepted windows.

## 10.3 Branch-B interpretation

The gate removes many wall-like or morphologically irregular q histories, but it is not guaranteed to preserve every real void. It is a selectivity experiment. A failure of Branch B can mean either “no coherent low-order ellipse” or “a physically real family was excluded by the gate.” Therefore Branch A remains the broader reference branch and Branch B is interpreted as a higher-selectivity subset.

# 11. Controls, recurrence, signed-depth equivalence, and ghosts

## 11.1 Full-support W = 50 control

The W50 inversion uses the same real two-component steering model but requires one depth family to fit the entire K=50 aperture. It is a stringent coherence control. Failure under W50 does not veto a W25 candidate because the canonical hypothesis explicitly allows partial-aperture coherence.

## 11.2 Raw Y and historical q_old controls

Depth inversion may be repeated on raw Y and on the retired two-focal-flank residual q_old. These are ablation controls. They are not substituted for q_arc in the canonical result.

## 11.3 Signed-depth equivalence

For the real cos/sin model, changing z to −z changes the sign of the sine column but does not change the two-dimensional column space:

span{cos(Kz z), sin(Kz z)} = span{cos(−Kz z), sin(−Kz z)}.    (58)

Therefore signed depth is not identifiable in this model. The primary physical search is unsigned.

## 11.4 Depth recurrence

If Kz is approximately sampled on a lattice Kz,k≈K0+kΔKz, the steering subspace repeats after a depth increment approximately

Z_rec ≈ 2π/|ΔKz|.    (59)

Because +z and −z are equivalent in the real subspace, a convenient unique unsigned interval is approximately

0 ≤ z < Z_rec/2 ≈ π/|ΔKz|.    (60)

For nonuniform Kz, the recurrence is estimated numerically from steering-column similarity rather than forced from a single ΔKz.

## 11.5 Ghost/alias rule

A single accepted W25 family can generate more than one local maximum along z because finite and quasi-periodic steering is not orthogonal over all candidate depths. Therefore repeated shallow/deep peaks are not automatically independent structures. A deeper peak is treated as an ambiguity/ghost candidate when it is generated by the same accepted W25 family as a stronger shallower maximum and lacks independent neighboring-window or spatial support.

Practical rule. Do not count two z peaks as two structures merely because they appear as separate colors in a tomogram. First compare the winning W25 identity, modal fingerprint, and neighboring spatial persistence.

# 12. Current frozen implementation values

The following values describe the present Great Pyramid / Capella working implementation. They are included to make the mathematical specification executable, but acquisition-dependent values must be recomputed for a different scene.

| Quantity | Current working value / rule |
| --- | --- |
| Current reference SLC | CAPELLA_C03_SP_SLC_HH_20210421192008_20210421192009; 3030×4565 complex raster. |
| Processed azimuth bandwidth | ≈3546.677 Hz. |
| Processed azimuth sample rate | ≈4252.786 Hz. |
| Radar center frequency | ≈9.65 GHz; radar carrier wavelength ≈0.03107 m. |
| Matched pair count | K=50. |
| Master/Offset broad-mask bandwidth | ≈1772.70 Hz in the frozen K50 implementation. |
| Master/Offset separation B_shift | 88.4242545 Hz. |
| Pair-center sweep | Approximately −837.164 to +837.164 Hz. |
| Registration support | 32×32 complex pixels; normalization=None; upsample_factor=100. |
| Arc nuisance sampling | 10 samples per side; at least 40 pixels from the entire target strip; 2D geometric median per side. |
| Canonical depth support | W=25 contiguous; 26 windows for K=50. |
| Branch A modes | m=1,…,24 over the full K50 q sequence. |
| Branch B modes | m=1,…,6 inside each W25 family. |
| Branch B gate | adjR²≥0.25; b/a≥0.10; b≥0.005 px; no phase-lock gate. |
| Effective inversion wavelength | λ_eff=0.2405679136690649 m in the current calibrated working model. |
| Approx. current unique unsigned depth interval | ≈0–232.4 m for the frozen geometry/model. |
| Primary corrected observable | q_arc = Y − G_arc. |

Do not conflate wavelengths. The X-band radar carrier wavelength controls the electromagnetic SAR phase history, while λ_eff in the current depth steering model is the declared operational wavelength used in Kz/depth calibration. The latter is empirical/model-dependent and must not be silently replaced by the radar wavelength.

# 13. End-to-end algorithms for the two branches

## 13.1 Common measurement core

Load and hash the current complex SLC/SICD. Verify axis mapping and metadata.

Freeze the target line(s) in native SLC coordinates and save the exact ordered rasterized pixels.

Build and freeze K=50 broad-overlap Master/Offset masks and ordered pair centers.

Reconstruct M_k and O_k for every k from the same SLC.

At every target position, register M_k against O_k with the 32×32 complex u-PCC estimator to obtain Y_k.

Measure the two nuisance arcs independently with the same estimator; compute G_+, G_− by 2D geometric medians and G_arc by side average.

Form q_k=Y_k−G_arc,k and save side-disagreement/scatter QC together with raw Y and historical controls.

Compute acquisition-specific Kz,k from state vectors and the declared λ_eff; audit recurrence and the unsigned search interval.

## 13.2 Branch A — full modal 1–24 + ungated W25 depth focus

For each spatial sample x, fit Eqs. (17)–(25) for every m=1,…,24 over all K=50 q vectors.

From the 2×2 coefficient matrix compute major/minor axes, b/a, orientation, handedness, RMS amplitude, and R²(b/a). Save all modes, not just the dominant one.

Optionally identify the descriptive dominant mode by Eq. (30); do not use that choice to discard W25 depth supports.

For every contiguous W25 support and every candidate unsigned depth z, perform Eqs. (37)–(43).

Form S_W25(x,z) as the maximum adjusted R² across all 26 windows and save the winning window identity.

Run W50, raw-Y, q_old, and complex signed-branch products only as controls/diagnostics.

## 13.3 Branch B — m = 1–6 ellipse-gated W25 depth focus

For each spatial sample x and each contiguous W25 support w, fit the local ellipse model in Eqs. (46)–(53) for m=1,…,6.

Choose the best m by maximum adjusted ellipse R².

Accept the W25 support only if the best mode also satisfies b/a≥0.10 and minor semiaxis b≥0.005 px, with adjusted ellipse R²≥0.25.

Pass each accepted W25 family, unchanged, into the same real two-component depth inversion of Eqs. (37)–(43).

At each x,z take the maximum depth adjusted R² over accepted windows only. Leave x blank where no W25 family passes.

Save gate occupancy, winning low-order mode, ellipse metrics, winning depth window, and full window-by-window proposals for interpreted candidates.

# 14. Interpretation and validation gates

The protocol deliberately separates numerical detection from physical identification. A strong mathematical fit is not by itself a void.

| Gate | Requirement |
| --- | --- |
| Axis/geometry gate | Verify the actual azimuth/Doppler-conjugate axis and native-SLC target geometry for the current acquisition. |
| Filter-freeze gate | Freeze Master/Offset masks and B_shift before inspecting target-dependent results. |
| Registration gate | Injected translations recover sign and subpixel scale; same-k pairing only. |
| Nuisance gate | Arc geometry is saved; G_arc identity and side-disagreement/scatter are audited. |
| Modal gate | Branch A and Branch B mode definitions are not mixed. Full K50 m=1–24 uses no intercept; local W25 m=1–6 fits a 2D center. |
| Depth-support gate | W=25 means exactly 25 contiguous pair indices. Winning support identity is saved. |
| Recurrence gate | Only the principal unsigned depth interval is interpreted as independent. |
| Ghost gate | Multiple z peaks from the same winning family are not counted as independent structures without independent support. |
| Spatial gate | Interpreted candidates should persist coherently over neighboring P samples and/or physically expected neighboring slice geometry. |
| Control gate | Masonry/surface controls, raw Y, W50, and slice-to-slice behavior remain visible. |
| Physical-depth gate | Absolute depth claims remain conditional on λ_eff and the image-registration forward model. |
| Motion-observable gate | q is an image-registration displacement observable; physical motion is a hypothesis requiring additional evidence. |

Completed audits already exclude several numerical explanations as the leading source of residual depth mismatch: intrinsic W25 least-squares drift, the implemented Kz/state-vector geometry, ordinary state-vector interpolation error, the mistaken tiny cross-orbit B⊥ interpretation, the retired two-flank subtraction as the sole cause, and a universal additive common-mode trajectory. Live explanations remain vector fidelity, residual local nuisance mismatch, wavelength/velocity-model mismatch, and true partial-aperture structure.

# Appendix A. Linear-algebra derivations

## A.1 Least-squares solution for a vector ellipse

For any design matrix H∈ℝ^{N×r} and observed vector matrix Q∈ℝ^{N×2}, the Frobenius-norm objective separates by output component:

min_C ||Q−HC||F² = min_{c_x}||q_x−Hc_x||₂² + min_{c_y}||q_y−Hc_y||₂².    (61)

When H has full column rank,

Ĉ = (H^T H)⁻¹ H^T Q = H⁺Q.    (62)

Thus the same design basis is fitted simultaneously to dx and dy while preserving their independent coefficient vectors.

## A.2 Why the harmonic fit traces an ellipse

Ignoring the center c, write the harmonic component as

q(θ) = E [cos θ, sin θ]^T.    (63)

The unit circle vector [cosθ,sinθ]^T is mapped by the linear transformation E into an ellipse. The singular-value decomposition E=UΣV^T rotates the unit circle by V^T, scales it by semiaxes a and b, and rotates it into the observed dx/dy plane by U. This is why the singular values of E are the ellipse semiaxes and why det(E) controls traversal handedness.

## A.3 Degrees of freedom used in adjusted R²

| Fit | Scalar observations n | Scalar fitted coefficients p | Current score |
| --- | --- | --- | --- |
| Full K50 modal m=1–24 | 2K = 100 | 4 (no intercept) | Stored ordinary R²; ellipse score R²(b/a). |
| W25 low-order ellipse gate | 2W = 50 | 6 (2D center + 4 harmonic coefficients) | Adjusted R² with p=6. |
| W25 depth fit | 2W = 50 | 4 (2×2 steering coefficient matrix) | Adjusted R² with p=4. |
| z=0 constant-vector diagnostic | 2W = 50 | 2 (constant dx,dy) | Separate diagnostic score. |

# Appendix B. Relationship between modal ellipses and depth ellipses

The two modal branches and the depth inversion all exploit the same geometric fact: a two-component vector generated by two quadrature basis functions lies on an ellipse in the dx/dy plane. They differ in what determines the phase.

| Analysis | Phase law | Question |
| --- | --- | --- |
| Full m=1–24 modal branch | θₖ=2πmk/K | Which harmonic of ordered pair index best describes the full K50 vector sequence? |
| Low-order W25 gate | ϑ_n=2πmn/W | Does this local W25 family form a well-sampled low-order ellipse? |
| Depth inversion | φₖ(z)=Kz,ₖ z | Does this W25 vector family lie in the steering subspace predicted by candidate depth z? |

A strong modal ellipse therefore does not determine depth by itself. Depth is assigned only when the same observed vector family is well represented by the acquisition-specific Kz steering at a candidate z. Conversely, a depth fit can be mathematically strong even when the full-aperture dominant modal order is not unique, because W25 permits partial-aperture support.

# Appendix C. Required output package and reproducibility manifest

Input SLC/SICD filename, cryptographic hash, raster shape, dtype, software versions, and axis mapping.

Exact target rasterized pixel order and all line-construction metadata.

K=50 pair centers, Master/Offset mask definitions, mask bandwidth, B_shift, and transform convention.

All target Y vectors and target registration errors.

All nuisance-arc sample vectors, arc center coordinates, G_+, G_−, G_arc, side disagreement, and within-side scatter.

q_arc vectors together with raw Y and q_old when control comparisons are required.

Scalar coherence/decoherence QC.

Branch-A modal table for every x and m=1,…,24, including R², axes, b/a, orientation, handedness, RMS amplitude, and ellipse score.

Kz geometry table, state-vector provenance, λ_eff declaration, recurrence estimate, and any independent Kz audit.

Canonical ungated W25 depth-score array, winning-window array, and per-window proposal table.

Branch-B gate table for every x,w,m=1,…,6; best-mode identity; pass/fail; gate occupancy; gated depth-score array; gated winning-window identity.

W50 stringent control, raw-Y control, historical q_old control when used, and complex signed/alias diagnostic.

Plotting scales. Comparative tomograms must use a common numerical color scale when their intensities are being visually compared.

Interpretation log distinguishing predicted-before-inspection targets from retrospective associations.

Version-control recommendation. Treat this document as a working mathematical specification layered on top of Protocol v1.4. If the m=1–6 branch is promoted from exploratory to canonical, issue a new numbered protocol version rather than silently editing v1.4.

# Compact mathematical summary

The protocol measures a two-component registration trajectory q across ordered broad-overlap sub-aperture pairs, removes a robust regional nuisance vector, characterizes harmonic ellipse structure, and searches candidate depth by fitting the q trajectory to the acquisition-specific two-column steering subspace [cos(Kz z), sin(Kz z)]. Branch A retains the complete m=1–24 full-aperture modal fingerprint and performs ungated W25 depth focusing. Branch B restricts each W25 support to a well-sampled low-order ellipse m=1–6 before allowing it to contribute to the identical W25 depth inversion. The two branches therefore differ in selectivity, not in the underlying depth steering model.