An April 2021 Capella Space Inc. Single-Look Complex Synthetic-Aperture RADAR (SLC-SAR) acquisition, comprising 3030 azimuth samples by 4565 range samples, provides a focused complex radar image in which every pixel retains complex amplitude and phase. The SLC is transformed into the spatial-frequency domain, with the azimuth dimension interpreted as the synthetic-aperture Doppler dimension. Two broad, strongly overlapping Doppler passbands—the Reference and Offset bands—are constructed, each spanning approximately half of the useful Doppler bandwidth and separated by a fixed relative displacement Bshift=88.424Hz. The paired filters are swept rigidly through the available Doppler spectrum at 50 positions, producing 50 matched Reference/Offset sub-aperture image pairs.

For every selected tomographic pixel position, the complex Reference and Offset images are co-registered using 32×32-pixel complex unnormalized phase correlation. This yields, at every sub-aperture position k, a two-component image-displacement vector

Yk=ΔxkΔyk,

so that each spatial pixel is represented not by a single scalar decoherence measure but by an ordered 50-state displacement-vector trajectory through aperture position.

Regional image deformation, speckle-driven displacement and other common-mode effects are estimated independently from two flanking arc-shaped reference fields surrounding the target pixel. The two-dimensional geometric median displacement is calculated for each arc and the two estimates are averaged to obtain a common-mode vector Gkarc. Subtracting this from the measured target displacement produces the localized residual trajectory

qk=Yk-Gkarc.

In the selective modal branch, every contiguous W=25segment of qkis tested for ordered elliptical harmonic motion according to

qn=c+ucos 2πmn25+vsin 2πmn25,

and only vector families satisfying the specified ellipse-quality criteria and having modal order m=1,…,6are retained. This produces a highly selective population of localized, structured displacement-vector families.

Each retained W=25 family is then tested against candidate depths using the aperture-dependent vertical wavenumber Kz,k. At each candidate depth z, the measured two-component vector trajectory is fitted to the real quadrature steering model

Xzk=cos⁡Kz,kz sin⁡Kz,kz.

A least-squares solution is calculated independently at every candidate depth. Depth focusing therefore occurs where the observed vector trajectory has maximum phase-consistent agreement with the predicted Kz-versus-depth evolution, equivalently minimum residual error and maximum adjusted R2. This inversion is performed over every contiguous, one-frame-shifted W=25 window, and the best surviving depth focus is retained for each spatial pixel.