The stages are:

- Raw complex SLC

Each pixel is a complex SAR sample

Sxy=Axyeiϕxy.

- Reference/Offset sub-aperture phasor fields

For each pair index k, filtering and inverse transformation produce two complex images:

Rkxy, Okxy.

These are still complex phasor fields. No displacement vector exists yet.

- Scalar coherence/decoherence branch

From the complex Rk,Oksamples we compute normalized complex coherence:

γk=∣OkRk*∣∣Ok∣2∣Rk∣2,

and

Dk=1-γk.

This is our scalar phasor-decoherence observable.

It does not produce dx,dy.

- Pairwise registration / pixel-tracking branch

Independently, we take a 32×32 complex patch from Rkand the matching patch from Ok, and phase-cross-correlate them.

The estimator returns the apparent subpixel shift

Yk=ΔxkΔyk.

This is what we call the raw target vector Yk.

It is “raw” only in the sense that no flank correction has yet been applied.

It is not raw phasor data.

- Flank measurements

We repeat exactly the same R/O registration at the two ±40-pixel perpendicular flank locations:

F+,k, F-,k.

Their average is

Ck=F+,k+F-,k2.

- Residual vector

We then define

qk=Yk-Ck.

So qkis the flank-corrected residual registration vector.

The terminology I would use from now on is:

| Stage | Name |
| --- | --- |
| Rk,Ok | complex sub-aperture phasor fields |
| γk,Dk | scalar coherence/decoherence |
| Yk=dxdy | raw target registration vector / apparent displacement vector |
| F±,k | flank registration vectors |
| Ck | flank-derived local common-mode estimate |
| qk=Yk-Ck | residual / flank-corrected vector |

And one especially important correction:

We do not globally co-register the 50-look stack.

For each k, phase cross-correlation is simply being used as a measurement operator to estimate the R/O displacement. Ykis the output of that registration measurement.

So the true chain is:

Rk,Ok phasors→u-PCC shift estimate→Yk→optional flank subtraction→qk

That distinction is important for what we are investigating now.